Answers

| Questions | Router 1 | Router 2 | VPCS 1 | VPCS 2 |
| --- | --- | --- | --- | --- |
| 1. Cấu hình cơ bản trên thiết bị: đặt IP như sơ đồ, đặt hostname, password console, password enable,... | enable configure terminal hostname R1 no ip domain-lookup enable secret 123 service password-encryption cdp run banner motd #Unauthorized access prohibited# line console 0 exec-timeout 0 password cisco login logging synchronous exit interface e0/0 ip address 192.168.12.1 255.255.255.0 no shutdown exit interface e0/1 ip address 192.168.1.1 255.255.255.0 no shutdown exit show ip interface brief | enable configure terminal hostname R2 no ip domain-lookup enable secret 123 service password-encryption cdp run banner motd #Unauthorized access prohibited# line console 0 exec-timeout 0 password cisco login logging synchronous exit interface e0/0 ip address 192.168.12.2 255.255.255.0 no shutdown exit interface e0/1 ip address 192.168.2.1 255.255.255.0 no shutdown exit show ip interface brief | ip 192.168.1.2/24 show ip | ip 192.168.2.2/24 show ip |
| 2. Thực hiện các lệnh "show cdp" trên các router để quan sát thông tin về các router láng giềng. | show cdp neighbors show cdp neighbors detail | show cdp neighbors show cdp neighbors detail | ping 192.168.1.1 | ping 192.168.2.1 |
| 3. Thực hiện cấu hình telnet trên các router sao cho: - VPC1 có thể telnet đến R1. - VPC2 có thể telnet đến R2. - R1 và R2 có thể telnet lẫn nhau. | configure terminal line vty 0 4 password cisco login transport input telnet end telnet 192.168.12.2 show users show sessions | configure terminal line vty 0 4 password cisco login transport input telnet end telnet 192.168.12.1 show users show sessions | rlogin 192.168.1.1 23 or telnet 192.168.1.1 | rlogin 192.168.2.1 23 or telnet 192.168.2.1 |
| 4. Thực hiện cấu hình SSH trên R1 và R2. | configure terminal ip domain-name local username admin privilege 15 secret cisco crypto key generate rsa How many bits in the modulus [512]: 2048 ip ssh version 2 line vty 0 4 login local transport input telnet ssh end show ip ssh show crypto key mypubkey rsa | configure terminal ip domain-name local username admin privilege 15 secret cisco crypto key generate rsa How many bits in the modulus [512]: 2048 ip ssh version 2 line vty 0 4 login local transport input telnet ssh end show ip ssh show crypto key mypubkey rsa | x | x |