Answers

| Questions | R1 | R2 | R3 | R4 | VPCS1 | VPCS2 | VPCS3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Thực hiện đấu nối dây giữa các thiết bị và đặt IP trên các cổng như hình. | enable configure terminal hostname R1 interface e0/0 ip address 192.168.123.1 255.255.255.0 no shutdown exit interface e0/1 ip address 192.168.14.1 255.255.255.0 no shutdown exit interface s1/0 ip address 192.168.13.1 255.255.255.0 no shutdown exit interface e0/2 ip address 172.16.1.1 255.255.255.0 no shutdown show ip interface brief | enable configure terminal hostname R2 interface e0/0 ip address 192.168.123.2 255.255.255.0 no shutdown exit interface e0/1 ip address 172.16.2.1 255.255.255.0 no shutdown show ip interface brief | enable configure terminal hostname R3 interface e0/0 ip address 192.168.123.3 255.255.255.0 no shutdown exit interface s1/0 ip address 192.168.13.3 255.255.255.0 no shutdown exit interface e0/1 ip address 172.16.3.1 255.255.255.0 no shutdown show ip interface brief | enable configure terminal hostname R4 interface e0/1 ip address 192.168.14.4 255.255.255.0 no shutdown exit interface loopback0 ip address 8.8.8.8 255.255.255.0 show ip interface brief | x | x | x |
| 2. Thực hiện hiệu chinh Router - ID như sau: • R1: 0.0.0.1 • R2: 0.0.0.2 • R3: 0.0.0.3 • R4: 0.0.0.4 Cấu hình các Router chạy OSPF đảm bảo mọi địa chỉ trong sơ đồ thấy được nhau. | router ospf 1 router-id 0.0.0.1 network 192.168.123.0 0.0.0.255 area 0 network 192.168.13.0 0.0.0.255 area 0 network 172.16.1.0 0.0.0.255 area 0 network 192.168.14.0 0.0.0.255 area 1 exit show ip route | router ospf 1 router-id 0.0.0.2 network 192.168.123.0 0.0.0.255 area 0 network 172.16.2.0 0.0.0.255 area 2 exit show ip route | router ospf 1 router-id 0.0.0.3 network 192.168.123.0 0.0.0.255 area 0 network 192.168.13.0 0.0.0.255 area 0 network 172.16.3.0 0.0.0.255 area 0 exit show ip route | router ospf 1 router-id 0.0.0.4 network 192.168.14.0 0.0.0.255 area 1 network 8.8.8.0 0.0.0.255 area 1 exit show ip route | x | x | x |
| 3. Thực hiện hiệu chỉnh quá trình bầu chọn DR/BDR đảm bảo: • Trong kết nối Multiple Access R1-R2-R3: R1, R3 là DR, R2 là BDR. • Trong kết nối Multiple Access R1-R4: R1 luôn luôn là DR. | interface e0/0 ip ospf priority 3 exit interface e0/1 ip ospf priority 2 end clear ip os process Reset ALL OSPF processes? [no]: (enter) show ip ospf neighbor | interface e0/0 ip ospf priority 2 end clear ip ospf process Reset ALL OSPF processes? [no]: (enter) show ip ospf neighbor | interface e0/0 ip ospf priority 3 end clear ip ospf process Reset ALL OSPF processes? [no]: (enter) show ip ospf neighbor | interface e0/1 ip ospf priority 0 end clear ip ospf process Reset ALL OSPF processes? [no]: (enter) show ip ospf neighbor | x | x | x |
| 4. Cấu hình Passive Interface trên các cổng đấu nối xuống PC tại R2 và R4. | x | configure terminal router ospf 1 passive-interface e0/1 exit show ip protocols | x | configure terminal router ospf 1 passive-interface loopback0 exit show ip protocols | x | x | x |
| 5. Cấu hình Default Route trên R3, quảng bá Default Route từ R3 sang các Router khác cho hoạt động đi Internet. | ping 192.168.123.2 ping 192.168.123.3 ping 192.168.13.3 ping 192.168.14.4 ping 172.16.1.2 ping 172.16.2.2 ping 172.16.3.2 ping 8.8.8.8 | ping 192.168.123.1 ping 192.168.123.3 ping 192.168.13.1 ping 172.16.1.2 ping 172.16.3.2 ping 192.168.14.4 ping 8.8.8.8 | configure terminal ip route 0.0.0.0 0.0.0.0 192.168.13.1 router ospf 1 default-information originate exit show ip protocols ping 192.168.123.1 ping 192.168.123.2 ping 192.168.13.1 ping 172.16.1.2 ping 172.16.2.2 ping 192.168.14.4 ping 8.8.8.8 | ping 192.168.14.1 ping 192.168.123.1 ping 192.168.123.2 ping 192.168.123.3 ping 172.16.1.2 ping 172.16.2.2 ping 172.16.3.2 | ip 172.16.1.2/24 172.16.1.1 show ip ping 172.16.1.1 ping 172.16.2.2 ping 172.16.3.2 ping 192.168.123.1 ping 192.168.123.2 ping 192.168.123.3 ping 192.168.14.4 ping 8.8.8.8 | ip 172.16.2.2/24 172.16.2.1 show ip ping 172.16.1.1 ping 172.16.2.2 ping 172.16.3.2 ping 192.168.123.1 ping 192.168.123.2 ping 192.168.123.3 ping 192.168.14.4 ping 8.8.8.8 | ip 172.16.3.2/24 172.16.3.1 show ip ping 172.16.3.1 ping 172.16.1.2 ping 172.16.2.2 ping 192.168.123.1 ping 192.168.123.2 ping 192.168.14.4 ping 8.8.8.8 |