Answers

| Questions | R1 | R2 | SW1 | SW2 | VPCS1 | VPCS2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Thực hiện đấu nối các thiết bị, thực hiện một số cấu hình cơ bản trên Router như đặt hostname, password console, password enable, telnet... | enable configure terminal no ip domain lookup hostname R1 enable secret class line console 0 exec-timeout 0 password cisco login logging synchronous exit username admin secret 123 line vty 0 4 login local transport input telnet exit service password-encryption exit configure terminal interface fa0/0 ip address 10.0.0.1 255.255.255.252 no shutdown exit interface fa0/1 ip address 192.168.1.1 255.255.255.0 no shutdown exit ip route 192.168.2.0 255.255.255.0 10.0.0.2 | enable configure terminal no ip domain lookup hostname R2 enable secret class line console 0 exec-timeout 0 password cisco login logging synchronous exit username admin secret 123 line vty 0 4 login local transport input telnet exit service password-encryption exit configure terminal interface fa0/0 ip address 10.0.0.2 255.255.255.252 no shutdown exit interface fa0/1 ip address 192.168.2.1 255.255.255.0 no shutdown exit ip route 192.168.1.0 255.255.255.0 10.0.0.1 | enable configure terminal hostname SW1 interface e0/0 switchport mode access no shutdown exit interface e0/1 switchport mode access no shutdown spanning-tree portfast exit | enable configure terminal hostname SW2 interface e0/0 switchport mode access no shutdown exit interface e0/1 switchport mode access no shutdown spanning-tree portfast exit | x | x |
| 2. Cấu hình DHCP: • Trường hợp 1: R1 làm DHCP server cấp IP cho PC1 và R2 làm DHCP server cấp IP cho PC2. • Trường hợp 2: R1 làm DHCP server cấp IP cho cả hai PC, R2 làm DHCP relay agent. | TH1: ip dhcp excluded-address 192.168.1.1 192.168.1.10 ip dhcp pool LAN1 network 192.168.1.0 255.255.255.0 default-router 192.168.1.1 dns-server 1.1.1.1 exit TH2: configure terminal ip dhcp excluded-address 192.168.1.1 192.168.1.10 ip dhcp excluded-address 192.168.2.1 192.168.2.10 ip dhcp pool LAN1 network 192.168.1.0 255.255.255.0 default-router 192.168.1.1 dns-server 1.1.1.1 exit ip dhcp pool LAN2 network 192.168.2.0 255.255.255.0 default-router 192.168.2.1 dns-server 1.1.1.1 exit show ip interface brief show ip route show ip dhcp binding | TH1: ip dhcp excluded-address 192.168.2.1 192.168.2.10 ip dhcp pool LAN2 network 192.168.2.0 255.255.255.0 default-router 192.168.2.1 dns-server 1.1.1.1 exit Th2: configure terminal interface fa0/1 ip helper-address 10.0.0.1 exit show ip interface brief show ip route show ip dhcp binding |  |  | TH1: dhcp show ip TH2: clear ip dhcp show ip ping 10.0.0.2 ping 192.168.1.1 ping 192.168.2.1 ping <IP_PC2> | TH1: dhcp show ip TH2: clear ip dhcp show ip ping 10.0.0.1 ping 192.168.2.1 ping 192.168.1.1 ping <IP_PC1> |