Answers

| Questions | SW1 | SW2 | SW3 | VPCS1 | VPCS2 | VPCS3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Thực hiện đấu nối các thiết bị và đặt hostname của các Switch như hình. | enable configure terminal no ip domain lookup hostname SW1 | enable configure terminal no ip domain lookup hostname SW2 | enable configure terminal no ip domain lookup hostname SW3 | x | x | x |
| 2. Sau khi thiết lập xong sơ đồ, tiếp tục tiến hành cấu hình thiết lập EtherChannel giữa SW1 và SW2 sử dụng giao thức LACP; thiết lập EtherChannel giữa SW2 và SW3 sử dụng giao thức PAgP và thiết lập EtherChannel giữa SW1 và SW3 không dùng giao thức. | interface range e0/0-1 channel-group 1 mode active no shutdown exit interface e0/2 switchport mode access switchport access vlan 10 spanning-tree portfast no shutdown exit show etherchannel summary | interface range e0/0-1 channel-group 1 mode active no shutdown exit interface range e0/2-3 channel-group 2 mode desirable no shutdown interface e1/0 switchport mode access switchport access vlan 10 spanning-tree portfast no shutdown exit show etherchannel summary | interface range e0/2-3 channel-group 2 mode auto no shutdown exit interface e0/0 switchport mode access switchport access vlan 10 spanning-tree portfast no shutdown exit show etherchannel summary | x | x | x |
| 3. Thiết lập đường EtherChannel đấu nối giữa các Switch thành đường trunk dot1q sau đó tạo thêm các VLAN 10, 20, 30 trên các Switch. | vlan 10,20,30 exit interface port-channel 1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk allowed vlan 10,20,30 exit show interface trunk | vlan 10,20,30 exit interface port-channel 1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk allowed vlan 10,20,30 exit interface port-channel 2 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk allowed vlan 10,20,30 exit show interface trunk | vlan 10,20,30 exit interface port-channel 2 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk allowed vlan 10,20,30 exit show interface trunk | x | x | x |
| 4. Hiệu chỉnh STP trên các Switch sao cho: a. VLAN10: SW1 làm root switch, SW2 làm secondary root switch. b. VLAN20: SW2 làm root switch c. VLAN30: SW3 làm root switch | spanning-tree mode pvst spanning-tree vlan 10 root primary show spanning-tree vlan 10 | spanning-tree mode pvst spanning-tree vlan 10 root secondary spanning-tree vlan 20 root primary show spanning-tree vlan 10 show spanning-tree vlan 20 | spanning-tree mode pvst spanning-tree vlan 30 root primary show spanning-tree vlan 30 | ip 192.168.1.1/24 ping 192.168.1.2 ping 192.168.1.3 show ip | ip 192.168.1.2/24 ping 192.168.1.1 ping 192.168.1.3 show ip | Ip 192.168.1.3/24 Ping 192.168.1.1 ping 192.168.1.2 show ip |