Answers

| Questions | Router | Switch | VPCS1 | VPCS2 | VPCS3 |
| --- | --- | --- | --- | --- | --- |
| 1. Thực hiện đấu nối các thiết bị và một số cấu hình cơ bản trên Switch và Router như đặt hostname, password console, password enable, telnet… | enable configure terminal hostname R no ip domain lookup enable secret 123 line console 0 password cisco login exec-timeout 0 exit username admin secret cisco line vty 0 4 login local transport input telnet exit | enable configure terminal hostname SW no ip domain lookup enable secret 123 line console 0 password cisco login exec-timeout 0 exit username admin secret cisco line vty 0 4 login local transport input telnet exit | ip 192.168.1.2/24 192.168.1.1 show ip | ip 192.168.2.2/24 192.168.2.1 show ip | ip 192.168.3.2/24 192.168.3.1 show ip |
| 2. Cấu hình định tuyến cho phép các PC khác VLAN giao tiếp được với nhau. | configure terminal interface Ethernet0/0 ip address 192.168.1.1 255.255.255.0 no shutdown exit interface Ethernet0/0.2 encapsulation dot1Q 2 ip address 192.168.2.1 255.255.255.0 exit interface Ethernet0/0.3 encapsulation dot1Q 3 ip address 192.168.3.1 255.255.255.0 exit show ip interface brief show running-config ¦ include hostname¦enable secret show running-config ¦ section line console | configure terminal vlan 1,2,3 exit interface vlan 1 ip address 192.168.1.251 255.255.255.0 no shutdown exit ip default-gateway 192.168.1.1 interface e0/0 switchport trunk encapsulation dot1q switchport mode trunk no shutdown exit interface e0/1 switchport mode access switchport access vlan 1 spanning-tree portfast no shutdown exit interface e0/2 switchport mode access switchport access vlan 2 spanning-tree portfast no shutdown exit interface e0/3 switchport mode access switchport access vlan 3 spanning-tree portfast no shutdown exit show ip interface brief show interface trunk show vlan brief show running-config ¦ include hostname¦enable secret show running-config ¦ section line console | ping 192.168.2.2 ping 192.168.3.2 | ping 192.168.1.2 ping 192.168.3.2 | ping 192.168.1.2 ping 192.168.2.2 |