Answers

| Questions | R1 | R2 | R3 |
| --- | --- | --- | --- |
| 1. Static route với outgoing interface đảm bảo mọi địa chỉ đi tới nhau. | enable configure terminal hostname R1 interface e0/0 ip address 192.168.123.1 255.255.255.0 no shutdown exit ip route 192.168.2.0 255.255.255.0 e0/0 ip route 192.168.3.0 255.255.255.0 e0/0 end show ip interface brief | enable configure terminal hostname R2 interface e0/0 ip address 192.168.123.2 255.255.255.0 no shutdown exit interface loopback0 ip address 192.168.2.1 255.255.255.0 exit ip route 192.168.3.0 255.255.255.0 e0/0 end show ip interface brief | enable configure terminal hostname R3 interface e0/0 ip address 192.168.123.3 255.255.255.0 no shutdown exit interface loopback0 ip address 192.168.3.1 255.255.255.0 exit ip route 192.168.2.0 255.255.255.0 e0/0 end show ip interface brief |
| 2. Khảo sát hoạt động của Proxy Arp trên R2 và R3. | Ping 192.168.123.1 Ping 192.168.123.2 Ping 192.168.123.3 ping 192.168.2.1 ping 192.168.3.1 + Khi tắt Proxy ARP không thể ping đến 2 địa chỉ Loopback0 show ip arp show ip route | configure terminal interface e0/0 no ip proxy-arp / ip proxy-arp end show ip arp show ip interface e0/0 | configure terminal interface e0/0 no ip proxy-arp / ip proxy-arp end show ip arp show ip interface e0/0 |