Answers

Cấu hình định tuyến SVI cho phép các PC khác VLAN giao tiếp với nhau sử dụng Switch layer 3

Switch:

enable

configure terminal

hostname SW

ip routing

vlan 2,3

exit

interface vlan 1

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface vlan 2

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface vlan 3

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

interface e0/1

switchport mode access

switchport access vlan 1

spanning-tree portfast

exit

interface e0/2

switchport mode access

switchport access vlan 2

spanning-tree portfast

exit

interface e0/3

switchport mode access

switchport access vlan 3

spanning-tree portfast

exit

// If VPCS can’t ping success: interface range e0/1-3 | shutdown | no shutdown

show ip interface brief

show vlan brief

show ip route

VPCS1

ip 192.168.1.2/24 192.168.1.1

ping 192.168.2.2

ping 192.168.3.2

VPCS2

ip 192.168.2.2/24 192.168.2.1

ping 192.168.1.2

ping 192.168.3.2

VPCS3

ip 192.168.3.2/24 192.168.3.1

ping 192.168.1.2

ping 192.168.2.2