Answer

| Questions | R1 | R2 | ISP | VPCS1 | VPCS2 |
| --- | --- | --- | --- | --- | --- |
| 1. Thực hiện đấu nối các thiết bị thiết lập một số cấu hình cơ bản trên Router như đặt hostname, password console, password enable,... | enable configure terminal hostname SG-Q1 no ip domain lookup enable secret cisco line console 0 password cisco login exec-timeout 0 exit interface e0/0 ip address 172.16.1.1 255.255.255.0 no shutdown exit interface e0/1 ip address 200.0.0.1 255.255.255.252 no shutdown exit show ip interface brief | enable configure terminal hostname SG-Q2 no ip domain lookup enable secret cisco line console 0 password cisco login exec-timeout 0 exit interface e0/0 ip address 172.16.2.1 255.255.255.0 no shutdown exit interface e0/1 ip address 11.1.1.1 255.255.255.252 no shutdown exit show ip interface brief | enable configure terminal hostname ISP no ip domain lookup interface e0/0 ip address 200.0.0.2 255.255.255.252 no shutdown exit interface e0/1 ip address 11.1.1.2 255.255.255.252 no shutdown exit ip route 0.0.0.0 0.0.0.0 200.0.0.1 ip route 0.0.0.0 0.0.0.0 11.1.1.1 ping 200.0.0.1 ping 11.1.1.1 | ip 172.16.1.2/24 172.16.1.1 ping 172.16.1.1 ping 172.16.12.2 ping 172.16.2.1 ping 172.16.2.2 ping 200.0.0.2 ping 11.1.1.1 show ip | ip 172.16.2.2/24 172.16.2.1 ping 172.16.2.1 ping 172.16.12.1 ping 172.16.1.1 ping 172.16.1.2 ping 200.0.0.1 ping 11.1.1.2 show ip |
| 2. Thiết lập VPN tunnel giữa 2 Router SG-Q1 và SG-Q2. Sử dụng định tuyến OSPF đảm bảo 2 PC ping thấy nhau. | interface tunnel0 ip address 172.16.12.1 255.255.255.0 tunnel source e0/1 tunnel destination 11.1.1.1 tunnel mode gre ip no shutdown exit ip route 0.0.0.0 0.0.0.0 200.0.0.2 router ospf 1 network 172.16.1.0 0.0.0.255 area 0 network 172.16.12.0 0.0.0.255 area 0 passive-interface e0/1 exit show ip route ping 172.16.1.2 ping 172.16.12.2 ping 172.16.2.1 ping 172.16.2.2 ping 200.0.0.2 ping 11.1.1.1 | interface tunnel0 ip address 172.16.12.2 255.255.255.0 tunnel source e0/1 tunnel destination 200.0.0.1 tunnel mode gre ip no shutdown exit ip route 0.0.0.0 0.0.0.0 11.1.1.2 router ospf 1 network 172.16.2.0 0.0.0.255 area 0 network 172.16.12.0 0.0.0.255 area 0 passive-interface e0/1 exit show ip route ping 172.16.2.2 ping 172.16.12.1 ping 172.16.1.1 ping 172.16.1.2 ping 200.0.0.1 ping 11.1.1.2 | x | x | x |
| 3. Áp dụng IPSEC Profile lên VPN tunnel để mã hóa dữ liệu trao đổi giữa 2 PС. | crypto isakmp policy 1 encryption aes 256 authentication pre-share group 24 exit crypto isakmp key Cisco address 11.1.1.1 crypto ipsec transform-set Cisco-trans esp-aes 256 esp-sha-hmac mode transport exit crypto ipsec profile CISCO set transform-set Cisco-trans exit interface tunnel0 tunnel protection ipsec profile CISCO exit show crypto isakmp policy show crypto isakmp sa show crypto ipsec sa show crypto session | crypto isakmp policy 1 encryption aes 256 authentication pre-share group 24 exit crypto isakmp key Cisco address 10.1.1.1 crypto ipsec transform-set Cisco-trans esp-aes 256 esp-sha-hmac mode transport exit crypto ipsec profile CISCO set transform-set Cisco-trans exit interface tunnel0 tunnel protection ipsec profile CISCO exit show crypto isakmp policy show crypto isakmp sa show crypto ipsec sa show crypto session | x | x | x |