Answers

| Questions | R1 | R2 |
| --- | --- | --- |
| 1. Học viên thực hiện kết nối cáp nối tiếp giữa hai Router như hình. | enable configure terminal hostname R1 interface s1/0 ip address 192.168.12.1 255.255.255.252 encapsulation ppp no shutdown exit show ip interface brief | enable configure terminal hostname R2 interface s1/0 ip address 192.168.12.2 255.255.255.252 encapsulation ppp no shutdown exit show ip interface brief |
| 2. Thực hiện chuyển giao thức lớp 2 trên liên kết nối tiếp giữa hai bộ định tuyến thành PPP và cấu hình để R1 xác thực PAP với R2 sử dụng tên người dùng “cisco" | username cisco password 123 interface s1/0 ppp authentication pap exit show interfaces s1/0 debug ppp authentication interface s1/0 shutdown no shutdown | interface s1/0 ppp pap sent-username cisco password 123 exit show interfaces s1/0 |
| 3. Bỏ cấu hình xác thực PAP đã thực thi. Thực hiện cấu hình để R1 xác thực CHAP với R2 sử dụng tên máy chủ của hai bộ định tuyến và mật khẩu chung là “cisco" | interface s1/0 no ppp authentication pap ppp authentication chap no username cisco This operation will remove all username related configurations with same name.Do you want to continue? [confirm] (enter) username R2 password cisco ping 192.168.12.2 | interface s1/0 no ppp pap sent-username cisco password cisco ppp authentication chap exit username R1 password cisco debug ppp authentication interface s1/0 shutdown no shutdown ping 192.168.12.1 |