Answers

| Questions | R1 | R2 |
| --- | --- | --- |
| 1. Thực hiện đấu nối và cấu hình cơ bản như sơ đồ. | enable configure terminal hostname R1 interface Loopback0 ip address 8.8.8.8 255.255.255.0 exit show ip interface brief | enable configure terminal hostname R2 interface Loopback0 ip address 1.1.1.1 255.255.255.0 exit show ip interface brief |
| 2. Thực hiện cấu hình PPPoE Server - Client với xác thực PAP và CHAP. | username cisco password 0 cisco username R2 password 0 cisco red bba-group pppoe Server virtual-template 1 ip local pool PPPoE_Pool 10.0.0.2 10.0.0.254 interface Virtual-Template 1 ip address 10.0.0.1 255.255.255.0 peer default ip address pool PPPoE_Pool ppp authentication pap chap exit interface e0/0 pppoe enable group Server no shutdown exit debug ppp authentication show pppoe session show ip interface brief show running-config interface Virtual-Template1 show running-config ¦ include username | interface Dialer0 ip address negotiated encapsulation ppp dialer pool 1 ppp pap sent-username cisco password 0 cisco ppp chap hostname R2 ppp chap password 0 cisco no shutdown exit interface e0/0 pppoe enable pppoe-client dial-pool-number 1 no shutdown exit debug ppp authentication show pppoe session show ip interface brief show running-config interface Dialer0 |
| 3. Cấu hình NAT đảm bảo mạng 1.1.1.1 ping thành công tới 8.8.8.8. | x | ip route 0.0.0.0 0.0.0.0 Dialer0 interface Dialer0 ip nat outside ex interface Loopback0 ip nat inside exit access-list 1 permit 1.1.1.0 0.0.0.255 ip nat inside source list 1 interface Dialer0 overload ping 8.8.8.8 source Loopback0 show ip route show ip nat translations show ip nat statistics |